Linux Router Lab

Full Network Configuration Guide

Client VM  •  Router VM  •  GNS3 Routers (R1, R2, R3)

# 1. Network Topology

This lab consists of three layers: a Client VM, a Router VM acting as a gateway, and three Cisco routers (R1, R2, R3) running inside GNS3.

┌─────────────┐         ┌──────────────────────┐         ┌─────────────┐

│  Client VM  │         │      Router VM       │         │  Internet   │

│ 192.168.10.2│──VMnet1─│ .10.1(ens224)        │──VMnet8─│  (via NAT)  │

│  Ubuntu     │         │ .62.132(ens224)      │         │             │

└─────────────┘         └──────────────────────┘         └─────────────┘

│ ens224 (VMnet1/Host-only)

┌──────────┴──────────┐

┌────┴────┐         ┌────┴────┐

│   R1    │  Switch  │   R2    │

│.62.2    │──────────│.62.3    │

└─────────┘    │     └─────────┘

┌────┴────┐

│   R3    │

│.62.4    │

## 1.1 IP Address Plan

| Device | Interface | IP Address | Subnet | Type |
| --- | --- | --- | --- | --- |
| Client VM (Ubuntu) | ens33 | 192.168.10.2 | /24 | Static |
| Router VM (RHEL) | ens224 | 192.168.10.1 | /24 | Static (LAN) |
| Router VM (RHEL) | ens224 | 192.168.62.132 | /24 | Static (GNS3) |
| Router VM (RHEL) | ens160 | DHCP | /24 | Dynamic (WAN) |
| R1 (GNS3) | e0/0 | 192.168.62.2 | /24 | Static |
| R2 (GNS3) | e0/0 | 192.168.62.3 | /24 | Static |
| R3 (GNS3) | e0/0 | 192.168.62.4 | /24 | Static |

## 1.2 VMware Network Design

| VMnet | Type | Subnet | Connected To |
| --- | --- | --- | --- |
| VMnet1 | Host-only | 192.168.10.x / 192.168.62.x | Router ens224 + Client ens33 |
| VMnet8 | NAT | 192.168.126.x | Router ens160 (WAN/Internet) |

# 2. Client VM Configuration (Ubuntu)

Run all commands on: navneet-VMware-Virtual-Platform

## 2.1 Temporary Config (ip commands)

Use these to set up the network immediately without rebooting:

sudo ip addr flush dev ens33

sudo ip addr add 192.168.10.2/24 dev ens33

sudo ip link set ens33 up

sudo ip route add default via 192.168.10.1

sudo ip route add 192.168.62.0/24 via 192.168.10.1

## 2.2 Permanent Config (Netplan)

Write the config file using tee (works without copy-paste):

sudo tee /etc/netplan/00-installer-config.yaml << 'EOF'

network:

version: 2

ethernets:

ens33:

dhcp4: false

addresses:

- 192.168.10.2/24

routes:

- to: default

via: 192.168.10.1

- to: 192.168.62.0/24

via: 192.168.10.1

nameservers:

addresses:

- 8.8.8.8

- 8.8.4.4

EOF

sudo chmod 600 /etc/netplan/00-installer-config.yaml

sudo netplan apply

## 2.3 Verify Client Config

ip a                        # check IPs

ip r                        # check routes

ping -c 4 192.168.10.1      # ping Router LAN

ping -c 4 192.168.62.2      # ping R1

ping -c 4 8.8.8.8           # ping Internet

# 3. Router VM Configuration (RHEL)

Run all commands on: navneet@localhost

## 3.1 Temporary Config (ip commands)

sudo ip addr flush dev ens224

sudo ip addr add 192.168.10.1/24 dev ens224

sudo ip addr add 192.168.62.132/24 dev ens224

sudo ip link set ens224 up

sudo dhclient ens160

## 3.2 Permanent Config (nmcli)

LAN Interface (ens224) — Static

sudo nmcli con mod ens224 ipv4.addresses '192.168.10.1/24 192.168.62.132/24'

sudo nmcli con mod ens224 ipv4.method manual

sudo nmcli con mod ens224 ipv4.gateway ''

sudo nmcli con mod ens224 connection.autoconnect yes

sudo nmcli con up ens224

WAN Interface (ens160) — DHCP

sudo nmcli con mod ens160 ipv4.method auto

sudo nmcli con mod ens160 connection.autoconnect yes

sudo nmcli con up ens160

## 3.3 Enable IP Forwarding (Permanent)

echo 'net.ipv4.ip_forward=1' | sudo tee /etc/sysctl.d/99-lab.conf

echo 'net.ipv4.conf.ens224.proxy_arp=1' | sudo tee -a /etc/sysctl.d/99-lab.conf

sudo sysctl -p /etc/sysctl.d/99-lab.conf

## 3.4 NAT Rules (Permanent)

sudo dnf install iptables-services -y

sudo systemctl enable --now iptables

sudo iptables -t nat -A POSTROUTING -o ens160 -j MASQUERADE

sudo iptables -A FORWARD -i ens224 -o ens160 -j ACCEPT

sudo iptables -A FORWARD -i ens160 -o ens224 -m state --state RELATED,ESTABLISHED -j ACCEPT

sudo service iptables save

## 3.5 Verify Router Config

ip a                              # check interfaces

ip r                              # check routes

cat /proc/sys/net/ipv4/ip_forward # should be 1

sudo iptables -t nat -L           # check NAT rules

ping -c 4 192.168.10.2            # ping Client

ping -c 4 192.168.62.2            # ping R1

# 4. GNS3 Routers Configuration (R1, R2, R3)

Open console for each router in GNS3 and run the commands below

## 4.1 R1 — 192.168.62.2

enable

configure terminal

interface ethernet 0/0

ip address 192.168.62.2 255.255.255.0

no shutdown

exit

ip route 192.168.10.0 255.255.255.0 192.168.62.132

end

write memory

## 4.2 R2 — 192.168.62.3

enable

configure terminal

interface ethernet 0/0

ip address 192.168.62.3 255.255.255.0

no shutdown

exit

ip route 192.168.10.0 255.255.255.0 192.168.62.132

end

write memory

## 4.3 R3 — 192.168.62.4

enable

configure terminal

interface ethernet 0/0

ip address 192.168.62.4 255.255.255.0

no shutdown

exit

ip route 192.168.10.0 255.255.255.0 192.168.62.132

end

write memory

## 4.4 Verify on Each GNS3 Router

show ip interface brief        # check interface status

show ip route                  # check routing table

ping 192.168.62.132            # ping Router VM

ping 192.168.10.2             # ping Client VM

# 5. Testing Sequence

Run tests in this order — each step confirms the previous layer works:

| Step | From | Command | Expected Result |
| --- | --- | --- | --- |
| 1 | Router VM | ping -c 4 192.168.10.2 | Client VM responds |
| 2 | Router VM | ping -c 4 192.168.62.2 | R1 responds |
| 3 | Router VM | ping -c 4 192.168.62.3 | R2 responds |
| 4 | Router VM | ping -c 4 192.168.62.4 | R3 responds |
| 5 | Client VM | ping -c 4 192.168.10.1 | Router LAN responds |
| 6 | Client VM | ping -c 4 192.168.62.2 | R1 responds via Router |
| 7 | Client VM | ping -c 4 192.168.62.3 | R2 responds via Router |
| 8 | Client VM | ping -c 4 192.168.62.4 | R3 responds via Router |
| 9 | Client VM | ping -c 4 8.8.8.8 | Internet works via NAT |
| 10 | R1/R2/R3 | ping 192.168.10.2 | Client VM responds |

# 6. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| Destination Host Unreachable | ARP incomplete — wrong VMnet | Match VMnet on both VM adapters |
| ping works Router→R1 but not Client→R1 | Missing route on Client or GNS3 router | Add route on Client and static route on R1/R2/R3 |
| ens224 gets wrong DHCP IP | VMnet assigning DHCP | Use nmcli to set static IP permanently |
| No internet from Client | IP forwarding off or NAT missing | Run sysctl and iptables commands on Router |
| GNS3 routers not reachable | Cloud1 not bound to correct interface | Bind Cloud1 to VMware Network Adapter VMnet1 |
| 100% packet loss, no errors | Firewall blocking ICMP | sudo systemctl stop firewalld on Router VM |
| Router VM loses IPs after reboot | Temp ip commands used | Use nmcli for permanent config |

# 7. Quick Reference Cheat Sheet

## Client VM Commands

| Command | Purpose |
| --- | --- |
| ip a | Show all IPs |
| ip r | Show routing table |
| sudo ip route add 192.168.62.0/24 via 192.168.10.1 | Add GNS3 route |
| ping -c 4 192.168.10.1 | Test Router LAN |
| ping -c 4 192.168.62.2 | Test R1 |
| sudo netplan apply | Apply netplan config |

## Router VM Commands

| Command | Purpose |
| --- | --- |
| ip a | Show interfaces |
| ip r | Show routes |
| sudo nmcli con show | List connections |
| sudo nmcli con up ens224 | Bring up LAN interface |
| sudo iptables -t nat -L | Check NAT rules |
| cat /proc/sys/net/ipv4/ip_forward | Check forwarding (should be 1) |
| sudo systemctl stop firewalld | Disable firewall for testing |

## GNS3 Router Commands

| Command | Purpose |
| --- | --- |
| show ip interface brief | Check interface status |
| show ip route | Check routing table |
| show running-config | Show full config |
| write memory | Save config to startup |
| ping 192.168.62.132 | Ping Router VM gateway |