Linux Router Lab

VMware Network Lab — Static Routing & NAT Configuration

RHEL / Ubuntu — Manual Configuration Guide

# 1. Lab Overview

This lab simulates a real-world network where a Client VM communicates with the internet through a Router VM. The Router VM acts as a gateway using NAT (Network Address Translation), forwarding traffic from the private LAN to the internet via VMware's NAT adapter.

## 1.1 Network Topology

[Client VM]  ──── VMnet1 (Host-only) ────  [Router VM]  ──── VMnet8 (NAT) ────  [Internet]

192.168.10.2                              192.168.10.1 | DHCP

ens224   |  ens160

## 1.2 Network Design

| Parameter | Value |
| --- | --- |
| LAN Network | 192.168.10.0/24 |
| Router LAN IP (ens224) | 192.168.10.1 |
| Client IP (ens33) | 192.168.10.2 |
| Router WAN IP (ens160) | DHCP from VMware NAT |
| Default Gateway (Client) | 192.168.10.1 |

# 2. VMware Network Settings

Correct VMware adapter configuration is critical. Both VMs must share VMnet1 for the LAN segment.

## 2.1 Router VM Adapters

| Adapter | VMware Type | Linux Interface | Purpose |
| --- | --- | --- | --- |
| Network Adapter 1 | NAT (VMnet8) | ens160 | WAN — Internet access via host |
| Network Adapter 2 | Host-only (VMnet1) | ens224 | LAN — Private network with Client |

## 2.2 Client VM Adapters

| Adapter | VMware Type | Linux Interface | Purpose |
| --- | --- | --- | --- |
| Network Adapter 1 | Host-only (VMnet1) | ens33 | LAN — Connects to Router's LAN |

⚠️  Important: Client VM must NOT have a NAT adapter. All internet traffic must route through the Router VM.

# 3. VMware VMnet Types Explained

| VMnet | Type | Internet | VM-to-VM | Physical Network | Used In This Lab |
| --- | --- | --- | --- | --- | --- |
| VMnet0 | Bridged | ✅ Yes | ✅ Yes | ✅ Yes | No |
| VMnet1 | Host-only | ❌ No | ✅ Yes | ❌ No | ✅ LAN segment |
| VMnet8 | NAT | ✅ Yes | ✅ Yes | ❌ No | ✅ WAN (Router) |
| VMnet2–7, 9+ | Custom | Configurable | ✅ Yes | Configurable | No |

# 4. Router VM Configuration

Run all commands on the Router VM (navneet@localhost).

## 4.1 Assign LAN IP to ens224

sudo ip addr flush dev ens224

sudo ip addr add 192.168.10.1/24 dev ens224

sudo ip link set ens224 up

## 4.2 Ensure WAN Interface Gets DHCP

sudo dhclient ens160

## 4.3 Enable IP Forwarding

This allows the Router to pass packets between ens224 (LAN) and ens160 (WAN):

sudo sysctl -w net.ipv4.ip_forward=1

## 4.4 Configure NAT (Masquerade)

These iptables rules enable NAT so the Client's private IP is translated to the Router's WAN IP:

sudo iptables -t nat -A POSTROUTING -o ens160 -j MASQUERADE

sudo iptables -A FORWARD -i ens224 -o ens160 -j ACCEPT

sudo iptables -A FORWARD -i ens160 -o ens224 -m state --state RELATED,ESTABLISHED -j ACCEPT

## 4.5 Verify Router Config

ip a

ip route

cat /proc/sys/net/ipv4/ip_forward   # should output: 1

sudo iptables -t nat -L

# 5. Client VM Configuration

Run all commands on the Client VM (navneet-VMware-Virtual-Platform).

## 5.1 Assign LAN IP to ens33

sudo ip addr flush dev ens33

sudo ip addr add 192.168.10.2/24 dev ens33

sudo ip link set ens33 up

## 5.2 Set Default Route via Router

sudo ip route add default via 192.168.10.1

## 5.3 Persist Config via Netplan (Ubuntu)

Edit /etc/netplan/00-installer-config.yaml:

network:

version: 2

ethernets:

ens33:

dhcp4: false

addresses:

- 192.168.10.2/24

routes:

- to: default

via: 192.168.10.1

nameservers:

addresses:

- 8.8.8.8

sudo netplan apply

# 6. Testing & Verification

## 6.1 Test Steps (Run from Client VM)

- Ping Router LAN IP (basic connectivity)

ping -c 4 192.168.10.1

- Ping internet (tests NAT and forwarding)

ping -c 4 8.8.8.8

- Check ARP table

arp -n

## 6.2 Traffic Flow — Client pings 8.8.8.8

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Client sends packet | src=192.168.10.2 dst=8.8.8.8 |
| 2 | Packet hits Router ens224 | Router receives on LAN interface |
| 3 | IP forwarding | Kernel forwards packet to ens160 |
| 4 | NAT Masquerade | src IP rewritten to Router's WAN IP |
| 5 | Packet exits to internet | Goes via VMnet8 → VMware NAT → Host |
| 6 | Reply arrives at Router | dst = Router's WAN IP |
| 7 | NAT reverse translation | dst rewritten back to 192.168.10.2 |
| 8 | Reply forwarded to Client | Via ens224 → VMnet1 → Client |

# 7. Common Issues & Fixes

| Symptom | Cause | Fix |
| --- | --- | --- |
| Destination Host Unreachable | ARP incomplete — no device at gateway IP | Check VMnet assignment on both VMs |
| ens224 gets 192.168.62.x IP | Wrong VMware network (not VMnet1) | Set adapter to Host-only (VMnet1) |
| No ping to 192.168.10.1 | VMnet mismatch between VMs | Both VMs must use VMnet1 for LAN adapter |
| No internet from Client | IP forwarding off or NAT missing | Run sysctl and iptables commands on Router |
| ens224 loses IP after reboot | IP set temporarily with ip command | Use nmcli or netplan to persist config |
| 100% packet loss, no errors | Firewall blocking ICMP | sudo systemctl stop firewalld (Router) |

# 8. Quick Reference

| Command | Purpose |
| --- | --- |
| ip a | Show all interfaces and IPs |
| ip r | Show routing table |
| arp -n | Show ARP table (check for 'incomplete') |
| ping -c 4 192.168.10.1 | Test LAN connectivity from Client |
| ping -c 4 8.8.8.8 | Test internet from Client |
| cat /proc/sys/net/ipv4/ip_forward | Verify IP forwarding is enabled (=1) |
| sudo iptables -t nat -L | List NAT rules on Router |
| sudo systemctl stop firewalld | Temporarily disable firewall for testing |